# Switch Abstraction Interface

# Change Proposal

| Title | Subport interface , .1D Bridges , bridge port interface |
| --- | --- |
| Authors | Mellanox |
| Status | In Review |
| Type | Standards Track |
| Created | 07/18/2016 |
| SAI-Version | 1.0 |

1	Overview	3

2	Sub port interface	3

2.1	New port type attribute	3

2.2	Example - Router sub port flow	4

3	Non vlan aware bridge domains (.1D Bridges)	4

3.1	Non Vlan aware bridge flows	5

4	Bridge ports	6

4.1	SAI bridge/router object current state	6

4.2	SAI bridge/router object proposed state	7

4.3	Bridge port type types	7

4.3.1	Bridge port type - Port	8

4.3.2	Bridge vport type – { Port , vlan}	8

4.3.3	Bridge port type- router	8

4.3.4	Bridge port type – tunnel	9

4.4	Bridge attributes	10

5	Specification	10

5.1	SAI port object	10

5.2	SAI FDB object	10

5.3	SAI STP object	11

5.4	SAI VLAN member object	11

5.5	SAI router interface object	11

5.1	SAI tunnel	12

5.2	SAI bridge port object	13

5.2.1	Bridge port  API summary	15

5.3	SAI bridge object	15

5.3.1	Bridge API summary	16

6	Examples	16

6.1	1.D bridge with router example	16

6.2	1.D bridge with VXlan  example	19

7	References	21

# Overview

The proposal contain three prats

- In order build a discrete pipeline and well define layers additional set of object are needed - Bridge ports. Currently when entering the switch we have -> ingress port /LAG, when entering a router we have SAI RIF. A bridge port object is missing.

- Increase SAI 4k vlan broadcast domain  by adding multiple vlan unaware bridges (.1D)  to the existing Vlan aware bridge (1.Q)

- Add an ability to define an interface base on {port,vlan} were interface can be router interface or bridge interface

# Sub port interface

Ability to define an interface base on {port,vlan} were interface can be router interface or bridge interface

## New port type attribute

Control on the port mode optional modes

- Port – legacy

- Sub port- port is split into logical interfaces, each interface is defined by {port,vlan}

## Example - Router sub port flow

# Non vlan aware bridge domains (.1D Bridges)

Increase SAI 4k vlan broadcast domain by adding multiple vlan unaware bridges (.1D) to the existing vlan aware bridge (1.Q)

Add ability to create Non vlan aware bridge object

Add ability to bind an "interface" to the a .1D bridge

- only interface type {port,vlan}

Please note that since PVID assignment is done prior to the interface classification untagged packets

can be classified based on {port,PVID}

## Non Vlan aware bridge flows

# Bridge ports

In order build a discrete pipeline and well defined layers, additional set of object are needed - Bridge ports. Currently when entering the switch we have -> ingress port /LAG, when entering a router we have SAI RIF. A bridge port object is missing.

## SAI bridge/router object current state

## SAI bridge/router object proposed state

## Bridge port type types

Port – represents phy port or LAG traditional .1Q bridge port

Vport - represents phy port or LAG vlan .1D bridge port interface

Router - represents the port that connects the bridge to the router

Tunnel - represents the port that connects the bridge to tunnel

.

### Bridge port type - Port

Represents phy port or LAG traditional .1Q bridge port

Can be added into .1Q Bridge only

Can be binded only form a port mode {port}

By default all ports will have a corresponding bride port added to the default 1.q bridge

Valid attribute:

### Bridge vport type – { Port , vlan}

Represents phy port or LAG .vlan

Can be added into .1D Bridge only

Can be binded only form a port mode {vport}

### Bridge port type- router

Represents the bridge port connection to the router

Two types of router bridge port

.1Q router bridge port- a single port representing all router Vlan interfaces

Per bridge .1D router bridge port per .1D bridge port

### Bridge port type – tunnel

Represents the bridge port connection to a tunnel

There is one to one mapping between the bride port and the tunnel

## Bridge attributes

All bridge related attribute will move from port/LAG to bridge port object

Bridge attribute

STP

Vlan objects

…

# Specification

## SAI port object

typedef enum _sai_port_bind_mode_attr_t

{

SAI_PORT_BIND_MODE_PORT,

SAI_PORT_BIND_MODE_SUB_PORT,

} sai_port_bind_mode_attr_t ;

typedef enum _sai_port_attr_t

{

//new attribute

/** port type[sai_port_bind_attr_t] */

SAI_PORT_BIND_MODE,

//removed atrtributes

SAI_PORT_ATTR_FDB_LEARNING,

SAI_PORT_ATTR_MAX_LEARNED_ADDRESSES,

SAI_PORT_ATTR_FDB_LEARNING_LIMIT_VIOLATION ,

} sai_port_attr_t ;

## SAI FDB object

typedef enum _ sai_fdb_entry_bridge_type_t

{

SAI_FDB_ENTRY_BRIDGE_TYPE_1Q,

SAI_FDB_ENTRY_BRIDGE_TYPE_1D,

} sai_fdb_entry_bridge_type_t ;

typedef struct _sai_fdb_entry_t

{

sai_mac_t mac_address;

sai_fdb_entry_br_type br_type; // .1D or .1Q

//valid for .1Q

sai_vlan_id_t vlan_id;

//valid for .1D

sai_bridge_id_t br_id;

} sai_fdb_entry_t;

typedef enum _sai_fdb_entry_attr_t

{

//new attribute

SAI_FDB_ENTRY_ATTR_BRIDGE_PORT_ID

//removed attributes

SAI_FDB_ENTRY_ATTR_PORT_ID

}

## SAI STP object

Set set port state- Panding STP port state – change to object

The port  state should be move from port to bridge port

typedef enum _sai_stp_attr_t

{

SAI_STP_ATTR_START,

/** READ-ONLY */

/** Vlans attached to STP instance [sai_vlan_list_t] */

SAI_STP_ATTR_VLAN_LIST = SAI_STP_ATTR_START,

//new attribute

/** bridge  attached to STP instance */

SAI_STP_ATTR_BRIDGE_ID

/** READ-WRITE */

SAI_STP_ATTR_END,

} sai_stp_attr_t;

## SAI VLAN member object

typedef enum _sai_vlan_member_attr_t {

SAI_VLAN_MEMBER_ATTR_START,

/** READ_WRITE */

/** VLAN ID [sai_vlan_id_t] (MANDATORY_ON_CREATE|CREATE_ONLY) */

SAI_VLAN_MEMBER_ATTR_VLAN_ID = SAI_VLAN_MEMBER_ATTR_START,

/** logical port ID [sai_object_id_t] (MANDATORY_ON_CREATE|CREATE_ONLY) */

SAI_VLAN_MEMBER_ATTR_PORT_ID,

/** logical bridge port  ID valid only for .1Q bridge ports  [sai_object_id_t] (MANDATORY_ON_CREATE|CREATE_ONLY) */

SAI_VLAN_MEMBER_ATTR_BRIDGE_PORT_ID,

/** VLAN tagging mode [sai_vlan_tagging_mode_t] (CREATE_AND_SET)

* (default to SAI_VLAN_PORT_UNTAGGED) */

SAI_VLAN_MEMBER_ATTR_TAGGING_MODE,

SAI_VLAN_MEMBER_ATTR_END,

/** custom range base value */

SAI_VLAN_MEMBER_ATTR_CUSTOM_RANGE_START = 0x10000000,

/* --*/

SAI_VLAN_MEMBER_ATTR_CUSTOM_RANGE_END

} sai_vlan_member_attr_t;

## SAI router interface object

/**

*  @brief Attribute data for SAI_ROUTER_INTERFACE_ATTR_TYPE

*/

typedef enum _sai_router_interface_type_t

{

/** Port or Lag Router Interface Type */

SAI_ROUTER_INTERFACE_TYPE_PORT,

/** VLAN Router Interface Type */

SAI_ROUTER_INTERFACE_TYPE_VLAN,

/** Loopback Router Interface Type */

SAI_ROUTER_INTERFACE_TYPE_LOOPBACK

/** sub port  Interface Type */

SAI_ROUTER_INTERFACE_TYPE_SUB_PORT

/** .1D bridge Interface Type */

SAI_ROUTER_INTERFACE_TYPE_BRIDGE

} sai_router_interface_type_t;

typedef enum _sai_router_interface_attr_t

{

/** READ-ONLY */

/** Virtual router id [sai_object_id_t] (MANDATORY_ON_CREATE|CREATE_ONLY) */

SAI_ROUTER_INTERFACE_ATTR_VIRTUAL_ROUTER_ID,

/** Type [sai_router_interface_type_t]  (MANDATORY_ON_CREATE|CREATE_ONLY) */

SAI_ROUTER_INTERFACE_ATTR_TYPE,

/** Assosiated Port or Lag object id [sai_object_id_t]

*  (MANDATORY_ON_CREATE when SAI_ROUTER_INTERFACE_ATTR_TYPE == SAI_ROUTER_INTERFACE_TYPE_PORT or SUB_PORT | CREATE_ONLY)

*/

SAI_ROUTER_INTERFACE_ATTR_PORT_ID,

/** Assosiated Vlan [sai_vlan_id_t]

*  (MANDATORY_ON_CREATE when SAI_ROUTER_INTERFACE_ATTR_TYPE == SAI_ROUTER_INTERFACE_TYPE_VLAN or SUB_PORT | CREATE_ONLY)

*/

SAI_ROUTER_INTERFACE_ATTR_VLAN_ID,

## SAI tunnel

typedef enum _sai_tunnel_map_type_t

{

/** TUNNEL Map overlay ECN to underlay ECN (only valid for encap) */

SAI_TUNNEL_MAP_TYPE_OECN_TO_UECN = 0x00000000,

/** TUNNEL Map underlay ECN and overlay ECN to overlay ECN (only valid for decap) */

SAI_TUNNEL_MAP_TYPE_UECN_OECN_TO_OECN = 0x00000001,

/** TUNNEL Map VNI to VLAN ID */

SAI_TUNNEL_MAP_TYPE_VNI_TO_VLAN_ID = 0x00000002,

/** TUNNEL Map VLAN ID to VNI */

SAI_TUNNEL_MAP_TYPE_VLAN_ID_TO_VNI = 0x00000003,

/** TUNNEL Map VNI to Bridge IF */

SAI_TUNNEL_MAP_TYPE_VNI_TO_BRIDGE_IF = 0x00000004,

/** TUNNEL Map Bridge IF to VNI */

SAI_TUNNEL_MAP_TYPE_BRIDGE_IF_TO_VNI = 0x00000005,

/** Custom range base value */

SAI_TUNNEL_MAP_TYPE_CUSTOM_RANGE_BASE = 0x10000000

} sai_tunnel_map_type_t;

## SAI bridge port object

/**

*  @brief Attribute data for SAI_ROUTER_INTERFACE_ATTR_TYPE

*/

typedef enum _sai_bridge_port_type_t

{

/** Port or Lag   */

SAI_BRIDGE_PORT_TYPE_PORT,

/** {Port or Lag.vlan}   */

SAI_BRIDGE_PORT_TYPE_SUB_PORT,

/**  bridge router port  */

SAI_BRIDGE_PORT_TYPE_1Q_ROUTER,

/**  bridge router port  */

SAI_BRIDGE_PORT_TYPE_1D_ROUTER,

/**  bridge tunnel  port  */

SAI_BRIDGE_PORT_TYPE_TUNNEL,

} sai_bridge_port_type_t;

typedef enum _sai_bridge_port_attr_t

{

/** Type [sai_bridge_port_type_t]  (MANDATORY_ON_CREATE|CREATE_ONLY) */

SAI_BRIDGE_PORT_ATTR_TYPE,

/** Assosiated Port or Lag object id [sai_object_id_t]

*  (MANDATORY_ON_CREATE when  SAI_BRIDEG_PORT_ATTR_TYPE == SAI_BRIDGE_PORT_TYPE_PORT or   SAI_BRIDGE_PORT_TYPE_SUB_PORT  | CREATE_ONLY)

*/

SAI_BRIDGE_PORT_ATTR_PORT_ID,

/** Assosiated vlan object id [sai_object_id_t]

*  (MANDATORY_ON_CREATE when  SAI_BRIDEG_PORT_ATTR_TYPE ==  SAI_BRIDGE_PORT_TYPE_SUB_PORT  | CREATE_ONLY)

*/

SAI_BRIDGE_PORT_ATTR_VLAN_ID,

/** Assosiated rouer inerface object id [sai_object_id_t]

*  (MANDATORY_ON_CREATE when  SAI_BRIDEG_PORT_ATTR_TYPE == SAI_BRIDGE_PORT_TYPE_RIF   | CREATE_ONLY)

*/

SAI_BRIDGE_PORT_ATTR_RIF_ID,

/** Assosiated tunnel id [sai_object_id_t]

*  (MANDATORY_ON_CREATE when  SAI_BRIDEG_PORT_ATTR_TYPE == SAI_BRIDGE_PORT_TYPE_TUNNEL | CREATE_ONLY)

*/

SAI_BRIDGE_PORT_ATTR_TUNNEL_ID,

/** Assosiated bridge ID  [sai_object_id_t]

*  (CREATE_AND_SET defult invalid bridge )

*/

SAI_BRIDGE_PORT_ATTR_BRIDGE_ID,

//bridge port attribute defualt leraning enadle

SAI_BRIDGE_PORT_ATTR_FDB_LEARNING,

// defualt no limit

SAI_BRIDGE_PORT_ATTR_MAX_LEARNED_ADDRESSES,

SAI_BRIDGE_PORT_ATTR_FDB_LEARNING_LIMIT_VIOLATION ,

} sai_bridge_port_attr_t ;

/**

* @brief Create bridge port

*

* @param[out]  bridge_port_id

* @param[in] 	 attr_count number of attributes

* @param[in] 	 attr_list array of attributes

*

* @return  SAI_STATUS_SUCCESS on success

*          Failure status code on error

*/

typedef sai_status_t (*sai_create_bridge_port_fn)(

_Out_ sai_object_id_t* bridge_port_id,

_In_ uint32_t attr_count,

_In_ const sai_attribute_t *attr_list

);

/**

* @brief Remove bridge port

*

*  @param[in] bridge_port_id.

*

*  @return  SAI_STATUS_SUCCESS on success

*           Failure status code on error

*/

typedef sai_status_t (*sai_remove_bridge_port_fn) (

_In_  sai_object_id_t   bridge_port_id

);

/**

* @brief Set attribute for bridge port

*

* @param[in] bridge port id

* @param[in] attr attribute to set

*

* @return  SAI_STATUS_SUCCESS on success

*          Failure status code on error

*/

typedef sai_status_t (*sai_set_bridge_port_attribute_fn)(

_In_ sai_object_id_t  bridge_port_id,

_In_ const sai_attribute_t *attr

);

/**

* @brief  Get attributes of bridge port

*

* @param[in] bridge_port_id

* @param[in] attr_count  number of attributes

* @param[inout] attr_list  array of attributes

*

* @return SAI_STATUS_SUCCESS on success

*        Failure status code on error

*/

typedef sai_status_t (*sai_get_bridge_port_attribute_fn)(

_In_ sai_object_id_t   bridge_port_id ,

_In_ uint32_t attr_count,

_Inout_ sai_attribute_t *attr_list

);

### Bridge port  API summary

typedef struct  bridge_port_t

{

sai_create_bridge_port_fn         			create_bridge_port;

sai_remove_bridge_port_fn 				remove_bridge_port;

sai_set_bridge_port_attribute_fn  			set_bridge_port_attribute;

sai_get_bridge_port_attribute_fn  			get_bridge_port_attribute;

} sai_bridge_port_api_t;

## SAI bridge object

typedef enum _sai_bridge_type_t

{

SAI_BRIDGE_TYPE_1Q,

SAI_BRIDGE_TYPE_1D,

} sai_bridge_type_t;

typedef enum _sai_bridge_attr_t

{

/** SAI bridge type vlan_aware , non_vlan_aware [sai_bridge_type_t] */

SAI_BRIDGE_ATTR_TYPE,

} sai_bridge_attr_t ;

/**

* @brief Create bridge

*

* @param[out]  bridge_port_id

* @param[in] 	 attr_count number of attributes

* @param[in] 	 attr_list array of attributes

*

* @return  SAI_STATUS_SUCCESS on success

*          Failure status code on error

*/

typedef sai_status_t (*sai_create_bridge_fn)(

_Out_ sai_object_id_t* bridge_id,

_In_ uint32_t attr_count,

_In_ const sai_attribute_t *attr_list

);

/**

* @brief Remove bridge

*

*  @param[in] bridge_id.

*

*  @return  SAI_STATUS_SUCCESS on success

*           Failure status code on error

*/

typedef sai_status_t (*sai_remove_bridge_fn) (

_In_  sai_object_id_t   bridge_id

);

/**

* @brief Set attribute for bridge

*

* @param[in] bridge id

* @param[in] attr attribute to set

*

* @return  SAI_STATUS_SUCCESS on success

*          Failure status code on error

*/

typedef sai_status_t (*sai_set_bridge_attribute_fn)(

_In_ sai_object_id_t  bridge_id,

_In_ const sai_attribute_t *attr

);

/**

* @brief  Get attributes of bridge

*

* @param[in] bridge_id  bridge_id

* @param[in] attr_count  number of attributes

* @param[inout] attr_list  array of attributes

*

* @return SAI_STATUS_SUCCESS on success

*        Failure status code on error

*/

typedef sai_status_t (*sai_get_bridge_attribute_fn)(

_In_ sai_object_id_t   bridge_id ,

_In_ uint32_t attr_count,

_Inout_ sai_attribute_t *attr_list

);

### Bridge API summary

typedef struct  bridge_t

{

sai_create_bridge_fn         				create_bridge;

sai_remove_bridge_fn 				remove_bridge;

sai_set_bridge_attribute_fn  			set_bridge_attribute;

sai_get_bridge_attribute_fn  			get_bridge_attribute;

} sai_bridge_api_t;

# Examples

## 1.D bridge with router example

//configure port 3,4,5,6 to sub port mode

sai_attribute_t port_attr;

port_attr.id=SAI_PORT_BIND_MODE;

port_attr.value= SAI_PORT_BIND_MODE_SUB_PORT;

sai_set_port_attribute_fn(port_3,port_attr);

sai_set_port_attribute_fn(port_4,port_attr);

//create bridges

sai_object_id bridge1

sai_object_id bridge2

sai_attribute_t  bridge_attr_list[1];

bridge_attr_list[0].id=SAI_BRIDGE_ATTR_TYPE;

bridge_attr_list[0].value=SAI_BRIDGE_TYPE_1D;

sai_create_bridge_fn(&bridge1,1, bridge_attr_list);

sai_create_bridge_fn(&bridge2,1, bridge_attr_list);

//create vports and add vport to bridge

sai_object_id vport3.7,vport3.6,vport4.16.vport4.17;

sai_attribute_t  vport_attr_list[4];

vport_attr_list[0].id= SAI_BRIDEG_PORT_ATTR_TYPE;

vport_attr_list[0].value=SAI_BRIDGE_PORT_TYPE_SUB_PORT;

vport_attr_list[1].id= SAI_BRIDGE_PORT_ATTR_PORT_ID;

vport_attr_list[1].value=port3;

vport_attr_list[2].id= SAI_BRIDGE_PORT_ATTR_VLAN_ID;

vport_attr_list[2].value=vlan7;

vport_attr_list[3].id= SAI_BRIDGE_PORT_ATTR_BRIDGE_ID;

vport_attr_list[3].value=bridge1;

sai_create_bridge_port_fn(&vport3.7,4, vport_attr_list);

vport_attr_list[0].id= SAI_BRIDEG_PORT_ATTR_TYPE;

vport_attr_list[0].value=SAI_BRIDGE_PORT_TYPE_SUB_PORT;

vport_attr_list[1].id= SAI_BRIDGE_PORT_ATTR_PORT_ID;

vport_attr_list[1].value=port3;

vport_attr_list[2].id= SAI_BRIDGE_PORT_ATTR_VLAN_ID;

vport_attr_list[2].value=vlan6;

vport_attr_list[3].id= SAI_BRIDGE_PORT_ATTR_BRIDGE_ID;

vport_attr_list[3].value=bridge1;

sai_create_bridge_port_fn(&vport3.6,4, vport_attr_list);

//create rif  …

sai_object_id bridge1_rif, bridge2_rif;

sai_attribute_t  rif_attr_list[2];

rif_attr_list[0].id= SAI_ROUTER_INTERFACE_ATTR_TYPE;

rif_attr_list[0].value=SAI_ROUTER_INTERFACE_TYPE_BRIDGE;

rif_attr_list[1].id= SAI_ROUTER_INTERFACE_ATTR_VIRTUAL_ROUTER_ID;

rif_attr_list[1].value=VRF_default;

sai_create_router_if_fn(&bridge1_rif,2, rif_attr_list);

rif_attr_list[0].id= SAI_ROUTER_INTERFACE_ATTR_TYPE;

rif_attr_list[0].value=SAI_ROUTER_INTERFACE_TYPE_BRIDGE;

rif_attr_list[1].id= SAI_ROUTER_INTERFACE_ATTR_VIRTUAL_ROUTER_ID;

rif_attr_list[1].value=VRF_default;

sai_create_router_if_fn(&bridge2_rif,2, rif_attr_list);

//create bridge router port & add interface to bridge (connect the RIF to the bridge)

sai_object_id bridge1_rport,bridge2_rport;

vport_attr_list[0].id= SAI_BRIDGE_PORT_ATTR_TYPE;

vport_attr_list[0].value=SAI_BRIDGE_PORT_TYPE_1D_ROUTER;

vport_attr_list[1].id= SAI_BRIDGE_PORT_ATTR_RIF_ID;

vport_attr_list[1].value=bridge1_rif;

vport_attr_list[2].id= SAI_BRIDGE_PORT_ATTR_BRIDGE_ID;

vport_attr_list[2].value=bridge1;

sai_create_bridge_port_fn(&bridge1_rport,3, vport_attr_list);

vport_attr_list[0].id= SAI_BRIDGE_PORT_ATTR_TYPE;

vport_attr_list[0].value=SAI_BRIDGE_PORT_TYPE_1D_ROUTER;

vport_attr_list[1].id= SAI_BRIDGE_PORT_ATTR_RIF_ID;

vport_attr_list[1].value=bridge2_rif;

vport_attr_list[2].id= SAI_BRIDGE_PORT_ATTR_BRIDGE_ID;

vport_attr_list[2].value=bridge1;

sai_create_bridge_port_fn(&bridge2_rport,3, vport_attr_list);

## 1.D bridge with VXlan  example

//configure port 3,4,5,6 to sub port mode

sai_attribute_t port_attr;

port_attr.id=SAI_PORT_BIND_MODE;

port_attr.value= SAI_PORT_BIND_MODE_SUB_PORT;

sai_set_port_attribute_fn(port_3,port_attr);

sai_set_port_attribute_fn(port_4,port_attr);

//create bridges

sai_object_id bridge1

sai_object_id bridge2

sai_attribute_t  bridge_attr_list[1];

bridge_attr_list[0].id=SAI_BRIDGE_ATTR_TYPE;

bridge_attr_list[0].value=SAI_BRIDGE_TYPE_1D;

sai_create_bridge_fn(&bridge1,1, bridge_attr_list);

sai_create_bridge_fn(&bridge2,1, bridge_attr_list);

//create vports and add vport to bridge

sai_object_id vport3.7,vport3.6,vport4.16.vport4.17;

sai_attribute_t  vport_attr_list[4];

vport_attr_list[0].id= SAI_BRIDEG_PORT_ATTR_TYPE;

vport_attr_list[0].value=SAI_BRIDGE_PORT_TYPE_SUB_PORT;

vport_attr_list[1].id= SAI_BRIDGE_PORT_ATTR_PORT_ID;

vport_attr_list[1].value=port3;

vport_attr_list[2].id= SAI_BRIDGE_PORT_ATTR_VLAN_ID;

vport_attr_list[2].value=vlan7;

vport_attr_list[3].id= SAI_BRIDGE_PORT_ATTR_BRIDGE_ID;

vport_attr_list[3].value=bridge1;

sai_create_bridge_port_fn(&vport3.7,4, vport_attr_list);

vport_attr_list[0].id= SAI_BRIDEG_PORT_ATTR_TYPE;

vport_attr_list[0].value=SAI_BRIDGE_PORT_TYPE_SUB_PORT;

vport_attr_list[1].id= SAI_BRIDGE_PORT_ATTR_PORT_ID;

vport_attr_list[1].value=port3;

vport_attr_list[2].id= SAI_BRIDGE_PORT_ATTR_VLAN_ID;

vport_attr_list[2].value=vlan6;

vport_attr_list[3].id= SAI_BRIDGE_PORT_ATTR_BRIDGE_ID;

vport_attr_list[3].value=bridge1;

sai_create_bridge_port_fn(&vport3.6,4, vport_attr_list);

//create rif  …

sai_object_id bridge1_rif, bridge2_rif;

sai_attribute_t  rif_attr_list[2];

rif_attr_list[0].id= SAI_ROUTER_INTERFACE_ATTR_TYPE;

rif_attr_list[0].value=SAI_ROUTER_INTERFACE_TYPE_BRIDGE;

rif_attr_list[1].id= SAI_ROUTER_INTERFACE_ATTR_VIRTUAL_ROUTER_ID;

rif_attr_list[1].value=VRF_default;

sai_create_router_if_fn(&bridge1_rif,2, rif_attr_list);

rif_attr_list[0].id= SAI_ROUTER_INTERFACE_ATTR_TYPE;

rif_attr_list[0].value=SAI_ROUTER_INTERFACE_TYPE_BRIDGE;

rif_attr_list[1].id= SAI_ROUTER_INTERFACE_ATTR_VIRTUAL_ROUTER_ID;

rif_attr_list[1].value=VRF_default;

sai_create_router_if_fn(&bridge2_rif,2, rif_attr_list);

//create bridge router port & add interface to bridge (connect the RIF to the bridge)

sai_object_id bridge1_rport,bridge2_rport;

vport_attr_list[0].id= SAI_BRIDEG_PORT_ATTR_TYPE;

vport_attr_list[0].value=SAI_BRIDGE_PORT_TYPE_1D_ROUTER;

vport_attr_list[1].id= SAI_BRIDGE_PORT_ATTR_RIF_ID;

vport_attr_list[1].value=bridge1_rif;

vport_attr_list[2].id= SAI_BRIDGE_PORT_ATTR_BRIDGE_ID;

vport_attr_list[2].value=bridge1;

sai_create_bridge_port_fn(&bridge1_rport,3, vport_attr_list);

vport_attr_list[0].id= SAI_BRIDEG_PORT_ATTR_TYPE;

vport_attr_list[0].value=SAI_BRIDGE_PORT_TYPE_1D_ROUTER;

vport_attr_list[1].id= SAI_BRIDGE_PORT_ATTR_RIF_ID;

vport_attr_list[1].value=bridge2_rif;

vport_attr_list[2].id= SAI_BRIDGE_PORT_ATTR_BRIDGE_ID;

vport_attr_list[2].value=bridge1;

sai_create_bridge_port_fn(&bridge2_rport,3, vport_attr_list);

//create the Vxlan tunnel

//Create underlay VRF

sai_object_id VRF_underlay;

… (VRF creataion flow )

//Create vxlan RIF

sai_object_id underlay_rif;

sai_attribute_t  rif_attr_list[2];

rif_attr_list[0].id= SAI_ROUTER_INTERFACE_ATTR_TYPE;

rif_attr_list[0].value=SAI_ROUTER_INTERFACE_TYPE_LOOPBACK;

rif_attr_list[1].id= SAI_ROUTER_INTERFACE_ATTR_VIRTUAL_ROUTER_ID;

rif_attr_list[1].value=VRF_underlay;

sai_create_router_if_fn(&vxlan_RIF,2, rif_attr_list);

//create tunnel

sai_object_id vxlan_tunnel;

sai_attribute_t  tunnel_attr_list[4];

tunnel_attr_list[0].id= SAI_TUNNEL_ATTR_TYPE;

tunnel_attr_list[0].value= SAI_TUNNEL_VXLAN;

tunnel_attr_list[1].id= SAI_TUNNEL_UNDERLAY_INTERFACE;

tunnel_attr_list[1].value=underlay_rif;

tunnel_attr_list[2].id= SAI_TUNNEL_IP_VER;

tunnel_attr_list[2].value=ipv4;

tunnel_attr_list[3].id= SAI_TUNNEL_SRC_IP;

tunnel_attr_list[3].value=1.1.1.1;

sai_create_tunnel_fn(&vxlan_tunnel,4, tunnel_attr_list);

//create tunnel bridge ports and add ports to bridge

sai_object_id bridge1_tport,bridge2_tport;

vport_attr_list[0].id= SAI_BRIDEG_PORT_ATTR_TYPE;

vport_attr_list[0].value=SAI_BRIDGE_PORT_TYPE_TUNNEL;

vport_attr_list[1].id= SAI_BRIDGE_PORT_ATTR_TUNNEL_ID;

vport_attr_list[1].value= vxlan_tunnel;

vport_attr_list[2].id= SAI_BRIDGE_PORT_ATTR_BRIDGE_ID;

vport_attr_list[2].value=bridge1;

sai_create_bridge_port_fn(&bridge1_tport,3, vport_attr_list);

vport_attr_list[0].id= SAI_BRIDEG_PORT_ATTR_TYPE;

vport_attr_list[0].value=SAI_BRIDGE_PORT_TYPE_TUNNEL;

vport_attr_list[1].id= SAI_BRIDGE_PORT_ATTR_TUNNEL_ID;

vport_attr_list[1].value=vxlan_tunnel;

vport_attr_list[2].id= SAI_BRIDGE_PORT_ATTR_BRIDGE_ID;

vport_attr_list[2].value=bridge2;

sai_create_bridge_port_fn(&bridge2_tport,3, vport_attr_list);

// update the encap and decap tunnel mapper

sai_attribute_t  tunnel_mapper_attr;

tunnel_mapper_attr.id= SAI_TUNNEL_DECAP_MAPPER;

tunnel_mapper_attr.value={type-SAI_TUNNEL_MAP_TYPE_VNI_TO_BRIDGE_IF

map { vni1-> bridge1_tport

vni2-> bridge2_tport}

sai_set_tunnel_attribute_fn(vxlan_tunnel,&tunnel_mapper_attr);

tunnel_mapper_attr.id= SAI_TUNNEL_ENCAP_MAPPER;

tunnel_mapper_attr.value={type- SAI_TUNNEL_MAP_TYPE_BRIDGE_IF_TO_VNI

map { bridge1_tport->nvi1

bridge2_tport->vni2}

sai_set_tunnel_attribute_fn(vxlan_tunnel,&tunnel_mapper_attr);

# References